文章修改：

1. 公式（20）中的Δtbuffer=α∙vmaxamax 删掉，”where α is a safety factor set to 1.2”这句也删掉，后面添加Δtbuffer的取值方式：Based on preliminary real-robot experiments, (Δtbuffer)was set to (0.20 s) and kept fixed throughout all experiments.

Algorithm 3 Two-Stage Dynamic Grasping Planning Strategy    这个表格里的vmax，amax等删掉，tinter ← tf + α vmax/amax 这个公式也相应改成+Δtbuffer的描述

2. C.3)中的When tr(P k) exceeds a safety threshold or Esync does not converge, 改成：

To quantify the positional uncertainty, we use the 3×3 positional covariance submatrix Pp,k of Pk, corresponding to  [px,py,pz]T. Its trace represents the sum of the position variances. When tr(Pp,k)>τP or Esync does not converge, precision placement is temporarily aborted.  We set τP=4.0×10−4 m2, corresponding to an aggregate positional standard deviation of 0.020 m.

3. 正文中添加两个个参数表？（缺的参数都列在下面两个表里了）

Table X. Parameters Used in PX3C and CAOS

| Module | Parameter | Value | Selection / Source |
| --- | --- | --- | --- |
| PX3C | VoxelGrid leaf size | 0.005 m | Reconstructed from the experiment record and fixed for reproduction |
| PX3C | RANSAC distance threshold | 0.005 m | Reconstructed from the experiment record and fixed for reproduction |
| PX3C | RANSAC hypotheses | 500 hypotheses/frame | Fixed computational setting |
| PX3C | Candidate OBB trim fraction | 0.05 | Reconstructed PCA preprocessing setting |
| PX3C | Neighborhood (N(i)) radius | 0.03 m | Fixed local-neighborhood setting; minimum 3 neighboring points |
| PX3C | τthin | 2.0 | Reconstructed thin-slice feature threshold |
| PX3C | ϵprot | 0.005 m | Protrusion-feature normalization scale |
| PX3C | τprot | 0.2 | Reconstructed protrusion-feature threshold |
| PX3C | τnorm | 0.065 | Reconstructed normal-consistency threshold |
| CAOS | Euclidean clustering tolerance | 0.150 m | Fixed connected-component clustering setting |
| CAOS | Minimum cluster points | 500 points | Fixed connected-component filtering setting |
| CAOS | τin | 0.010 m | Inlier-distance threshold |
| CAOS | wp | 2.0 | Normalized squared position-change weight |
| CAOS | wr | 1.06 | Normalized squared rotation-change weight |
| CAOS | ws | 0.38 | Normalized squared scale-change weight |
| CAOS | η | 1.0 | Common normalization factor |
| CAOS | α1 | 0.43 | Weight of (qcov) |
| CAOS | α2 | 0.21 | Weight of (qin) |
| CAOS | α3 | 0.36 | Weight of (qstab) |
| CAOS | qmin | 0.65 | Low-quality observation threshold |
| CAOS | βmax | 0.70 | Maximum adaptive smoothing weight |
| CAOS | k | 2 | Adaptive smoothing coefficient |

Response:（供参考）

Thank you for pointing out that several important parameters in the 3D modeling module were not explicitly reported. We have added Table X to Section IV-B to provide the main PX3C and CAOS parameters, including preprocessing settings, geometric-filtering thresholds, clustering parameters, quality-assessment weights, and temporal-smoothing parameters. Parameters that do not have explicit mathematical symbols in the original formulation are reported using their implementation names.

Table XX. Main parameters of the DKF and motion planner

| Module | Parameter | Value | Selection / Source |
| --- | --- | --- | --- |
| DKF | Process-noise covariance (Q) | (diag([10−5,10−4,10−3]×3)) | Initialized according to the constant-acceleration model and adjusted through preliminary tracking experiments |
| DKF | Measurement-noise covariance(R) | (diag(2.5×10−5,2.5×10−5,1.0×10−4) m2) | Estimated from repeated static RGB-D measurements |
| DKF | Update interval (ΔT) | (0.20 s) | System operating setting |
| Coarse planning | Interception height (Δh) | (0.080 m) | Determined from the end-effector geometry and required clearance |
| Coarse planning | Average interception velocity (v) | (1.0 m/s) | Empirically selected within the feasible operating range |
| Coarse planning | Newton-Raphson tolerance | (1.0×10−3 s) | Numerical convergence setting |
| Coarse planning | Buffer time (Δtbuffer\\) | (0.20 s) | Empirically selected through preliminary real-robotbot experiments |
| Fine synchronization | Velocity reference (vref) | (0.4932 m/s) | Set to the conveyor-belt operating velocity |
| Fine synchronization | Position weight (λ) | (0.01) | Selected to balance the normalized velocity and position errors |
| Fine synchronization | Position reference (dref) | (0.020 m) | Set according to the required positioning accuracy |
| Fine synchronization | Synchronization threshold (ϵsafe) | (0.01) | Selected according to the acceptable residual velocity and position errors |
| Active regulation | Position-uncertainty threshold (τP) | (4.0×10−4 m2) | Empirically selected through preliminary validation |

Response（供参考）:

Thank you for pointing out that several important parameters of the discrete Kalman filter and motion planner were not explicitly reported. To address this issue, we have added a parameter table xx in Section IV-C of the revised manuscript. The parameters are listed in the same order as they appear in the method description, together with their values, units, and selection methods.

The DKF process-noise covariance was initialized according to the constant-acceleration motion model and adjusted through preliminary tracking experiments, whereas the measurement-noise covariance was estimated from repeated static measurements of the RGB-D sensor. The operating-condition and geometry-related parameters were determined by the system configuration. The remaining planning parameters were selected through preliminary real-robot experiments and then kept fixed throughout the final evaluation.

For the synchronization cost,  vref=0.4932 m/s corresponds to the conveyor-belt velocity, while dref=0.020 m corresponds to the required positioning accuracy. With λ=0.01 and ϵsafe=0.01, the synchronization condition corresponds to a maximum residual velocity of approximately 0.04932 m/s when the position error is zero, or a maximum planar position error of 0.020 m when the velocity error is zero. When both errors are nonzero, they are jointly constrained by the normalized synchronization cost.

We have also clarified the buffer-time setting used in the coarse-planning stage. The heuristic expression Δtbuffer=αvmax/amax in the previous version has been removed. Instead, the actual buffer time used in the experiments,(Δtbuffer=0.20 s), is now explicitly reported as an empirically selected parameter. Accordingly,α,vmax, and amaxhave been removed from the method description, Algorithm 3, and the parameter table.